# Technical Reference Manual: V+ Command Protocols for Adept Robot Systems

## 1. System Initialization: Calibrating and Loading Programs from Disk/Floppy

Upon successful reconciliation of the joint encoders via the CALIBRATE protocol, the V+ system establishes a reliable "Ground Truth." This sequence is a strategic necessity in industrial automation, serving as the critical bridge between the robot's incremental hardware encoders and the absolute World Coordinate System. Without this synchronization, the system remains "blind" to its own geometry, rendering hardware and software synchronization impossible.

### 1.1 Hardware Readiness and Calibration (CALIBRATE)

The CALIBRATE instruction is the fundamental command used to initialize the robot positioning system. This instruction reconciles physical joint positions with internal software models.

- Mode Requirements: For execution, the system must be in COMP (Computer) mode.

- Critical Dependencies: Calibration is effectively disabled if the DRY.RUN system switch is enabled, as this state prevents V+ from communicating with the underlying motion hardware.

- Workspace Safety: For Adept robots, CALIBRATE initiates specific physical movements. Consequently, the robot must be positioned far from workspace limits and obstacles. Until this process is complete, program control and Manual Control Pendant (MCP) motions are strictly prohibited by the operating system.

### 1.2 Software Loading and Execution (LOAD & EXECUTE)

Following hardware synchronization, the software environment must be populated. V+ transitions programs from storage—referenced as "DISK" or the "SYSTEM" device—into active system memory. The EXECUTE command performs a vital dual role: it initiates the program stack and automatically handles the ATTACH protocol for Robot #1. This ensures the program secures immediate, exclusive control of the primary motion hardware, provided the DRY.RUN switch is in the off state.

### 1.3 The "So What?" Layer: Calibration Integrity

Skipping calibration or failing to verify status via the STATE function or NOT.CALIBRATED parameter compromises the entire safety and precision profile of the workcell. Without calibration, the operating system locks out all program-driven motion and MCP functionality. From an architectural standpoint, calibration is the only method to ensure the sub-millimeter precision required for high-speed assembly and reliable error-trapping.

Once the system state is reconciled and the operating system is live, the user may transition to direct manual control or real-time debugging via the terminal.

--------------------------------------------------------------------------------

## 2. Direct Robot Control via ASCII Terminal (Monitor Commands)

The ASCII terminal serves as the primary human-machine interface (HMI) for real-time system state management. It provides a low-latency environment for manual overrides, coordinate "teaching," and direct hardware interrogation.

### 2.1 Establishing Control (SELECT & ATTACH)

Manual control requires explicit hardware allocation. The SELECT command identifies the active robot unit (e.g., Robot #1). Once selected, the ATTACH instruction secures exclusive control of the motion hardware for the terminal task. This prevents background program tasks from interfering with manual adjustments during the deployment phase.

### 2.2 Motion and Configuration Commands (MOVE, APPRO, SPEED)

V+ motion commands vary based on the required trajectory. Use the following protocols for terminal-based motion:

| Command Name | Motion Type | Primary Parameter |
| --- | --- | --- |
| MOVE | Joint-interpolated | Location/Transformation |
| MOVES | Straight-line | Location/Transformation |
| APPRO | Joint-interpolated | Location + Distance offset |
| APPROS | Straight-line | Location + Distance offset |

The SPEED command defines nominal motion factors. Appending the ALWAYS keyword extends the instruction's duration, making the specified speed or configuration (such as COARSE or FINE) permanent for all subsequent motions until a new instruction is issued.

### 2.3 Position Recording and Querying (HERE & STATUS)

Capturing coordinates is essential for path programming. The HERE command records the robot’s current physical location into a transformation or precision-point variable. To monitor the health of an active program task, the STATUS real-valued function provides an immediate query of the task’s execution state.

### 2.4 The "So What?" Layer: Terminal Efficiency

Utilizing monitor commands like ALIGN (which squares the Tool Z-axis to the nearest world axis) or WHERE significantly streamlines the "teaching" phase. By executing these via the DO command, a systems architect can verify spatial geometry and verify tool orientation before committing to permanent code, reducing overall deployment cycle times.

--------------------------------------------------------------------------------

## 3. External Integration: Serial Port Communication and Remote Execution

In multi-device environments, serial I/O allows the robot to function as either a master node, commanding peripherals, or a slave node, receiving factory-level PLC instructions.

### 3.1 Port Configuration and Device Allocation (ATTACH & FSET)

V+ utilizes Logical Unit Numbers (LUNs) to map hardware devices. If no $device string is provided, V+ uses the following default mappings:

| LUN | Default Mapping | Hardware Interface |
| --- | --- | --- |
| 1 | Manual Control Pendant (MCP) | Global Serial 4 |
| 2 - 4 | System Terminal | System Processor |
| 5 - 8 | Disk Devices | Disk Controller |
| 10 - 13 | Global Serial 1 - 4 | SIO Board |
| 14 - 15 | Local Serial 1 - 2 | System Processor |
| 20 - 24 | Graphics Windows | System Processor |

The ATTACH instruction must be executed to claim the port before FSET can modify baud rates or parity. Note that a subsequent ATTACH will reset the port to its boot-default parameters.

### 3.2 Data Transmission and Programmatic Control (READ, WRITE, IOSTAT)

To process external strings, the integration workflow is as follows:

- ATTACH a LUN to the serial device.

- FOPEN the device for input/output operations.

- READ the ASCII string from the external terminal or controller.

- IOSTAT the LUN to monitor the success of the transfer (positive values indicate success).

A running program can interpret these strings to trigger motion via the DOS instruction. However, from a systems architecture perspective, executing arbitrary strings received over serial lines poses significant security and stability risks; robust error-trapping and string validation are mandatory.

### 3.3 Logical Operations and Signal Handling (BITS, SIG)

The BITS instruction allows the robot to communicate status codes back to an external controller by setting or clearing groups of up to 8 digital signals simultaneously. This is often used to signal "Task Complete" or "Error State" to a master PLC.

### 3.4 The "So What?" Layer: The Power of Asynchronous Integration

Mastery of "No-wait" I/O (activated via Mode Bit 2 in the ATTACH instruction, noted as ^B010) is critical for high-speed integration. This mode prevents "mechanical dwell," allowing the robot to maintain fluid motion while simultaneously processing data streams. By decoupling communication latency from the mechanical cycle, architects ensure maximum throughput in the automated cell.

--------------------------------------------------------------------------------

## Appendix: Critical Troubleshooting and Help Protocols

### Error Identification

Before contacting Adept Service, the following diagnostic data must be retrieved using the ID command:

- System Identification: Software version and Controller serial number.

- Hardware Data: Robot serial number and the value of the HOUR.METER.

- System State: The exact error message and the results of the diagnostic tests listed below.

### Hardware Diagnostic Flowchart Tests

If motion or feedback errors occur, perform the following hardware-level verifications:

- Encoder Activity Test: Display joint values on the MCP; values should update when joints are moved manually.

- 100 Ohm Test: Use an ohm meter to check encoder lines; they should show 100 ohms of resistance.

- JIB Swap Test: Swap the Joint Interface Boards (JIB) in the card cage. If the error moves to a different joint, the JIB is faulty.

- Solenoid +12 Vdc Test: If the Robot System Controller (RSC) is functional, 12 Vdc must be present across the pins of J1.

- Voltage Verification: Check for 110 Vac at the amplifier inputs and verify that 12 and 24 Vdc from the Power Supply (PSS) are acceptable.

- JIB-to-OLC Check: If errors persist on Joints 1 or 2 after a JIB swap, the issue likely resides in the RSC. For Joints 3 or 4, check the OLC (Out-board Link Controller).

Adept Service Contact (USA): (800) 232-3378 (24/7 Support)