V and V+

Version 10

Quick Reference Guide

adept

adept technology, inc.

TABLE OF CONTENTS

The information contained herein is the property of Adept Technology, Inc. and shall not be reproduced in whole or in part without prior written approval of Adept Technology, Inc. The information herein is sub- ject to change without prior notice and should not be construed as a commitment by Adept Technology, Inc. This document is periodically reviewed and revised to accurately reflect and incorporate changes made.

Adept Technology, Inc. assumes no responsibility for any errors or omissions in this document. Users' critical evaluation of this document is welcome to assist in the preparation of future documentation.

Product specifications are subject to change without notice. Adept, AdeptVision, V and V+ are trademarks of Adept Technology, Inc.

Program Edit and Debug

SEE Editor

Line Editor

Program Instructions Monitor Commands Functions

Real Valued Functions Precision-Point Functions String Functions TransformationFunctions

Switches Parameters Operators

About this Quick Reference

2

3

4

16

22

26

26

2 ** 722

29

32

35

This guide provides the syntax and a concise description of Adept Technology's V and V+ version 10.1 languages and operating systems. The purpose of this guide is to provide informational assistance only. The user should refer to Adept's V and V+, or AdeptVision Reference Guides with any questions as to syntax, description, or usage.

The following is a summary of the conventions used in this document when referencing Adept keywords:

•BOLD/CAPITALIZED words are actual keywords uncapitalized words are conditions or arguments

• italicized words are optional arguments

• $ indicates the argument has a string value

The following abbreviations refer to the type of keyword:

PI

Program Instruction

MC Monitor Command

RVF Real Valued Function

Precision-Point Function

PPF

SF

Copyright © 1991 by Adept Technology, Inc. All rights reserved.

String Function

TF

Transformation Function

SW Switch

PAR Parameter

V and V+ Quick Reference Guide

1

PROGRAM EDITING AND DEBUG

SEE Editor

SEE DEBUGGER COMMANDS

Keystroke

SEE Description

лв

ΔΕ

SEE program_name [R

MC

^G

initiate screen-oriented editing of a new or existing program (/R optional)

^N

пр

SEE(logical_unit) program_spec, step

Pl

^X

initiate screen-oriented editing of a program

^Z

MC

DEBUG task_number program_name

invoke program debugger to allow a program to be executed and viewed simultaneously

SEE EDITOR COMMANDS

Label

New

Go To

Equivalent Command N

SEE Description

New program

Goto subroutine

Goto home program

Exit

Display value

Teach value

Undo line edit Redraw

Find setup Change setup Repeat find Copy one line

Set breakpoint at cursor line Alternate between modes Execute at cursor line

Clear breakpoint at cursor line Proceed execution

Execute single instruction

Execute instruction or subroutine

Line Editor

EDIT program_name, step

initiate line-oriented editing of a new or existing program

Line Editor Commands

Any program instruction

replace the current instruction

Carriage return

leave the current line unaltered and move to

the next line

C program, step

change to editing a new program

D steps

delete a number of program steps

Retrieve Exit Display

Esc E

Teach

Undo

Esc ^C

Redraw

V

Find

F

Change

C

Repeat

Copy

A

Cut

Esc A

Cut one line

Paste

-Esc A

Paste one line

exit the editor and return to the monitor

Paste All

E

Paste all

I

Edit

Edit Cmd mode

initiate line insert mode

Debug

Debug mode

L

Replace

R

Replace mode

display the previous program step

Insert

I

Insert mode

Ins Line

Esc I

Insert line

Delete

D

Del Line

Esc D

Delete line

Top

Esc T

Top of program

Prog Up

Bottom

Esc B

Prog Down

Page Up

Page Down

Delete next character

Stack

up

in Debug

Bottom of program Stack down in Debug

Back one screen

Next screen

P steps

display a number of program steps

R, character_string

replace a portion of a program step

S step_number

skip to a specific program step

T location_variable

initiate teach mode

TS location_variable

initiate straight-line teach mode

Start

End

Start of line

End of line

2

Adept Technology, Inc.

V and V+ Quick Reference Guide

3

ABOVE

PROGRAM INSTRUCIONS

request change to elbow-up configuration during next motion [see BELOW(PI)]

ACCEL acceleration, deceleration

set acceleration and deceleration for subsequent motions

AMOVE axis = position

ANY

position an "extra" axis during the next joint- interpolated or straight-line motion

signal the begining of an alternative group instructions for the CASE structure

APPRO location, distance

of

move to a location offset from a named location APPROS location, distance

move to a location offset from a named location

in a straight-line motion with no configuration change

ATTACH (logical_unit, mode) $device

make a device available for use by the application program

AUTO type variable, variable

***

declare temporary variables that are automatically allocated on the program stack when the program is entered

BASE X_shift, Y _shift, Z_shift, Z_rotation

define the location of the robot base, relative

to its workspace [see BASE (LF,MC)]

BELOW

request change to elbow-down configuration during the next motion [see ABOVE(PI)]

BITS first signal, num_sigs = value

set the state of a group of digital output or software signals [see BITS (RVF,MC)]

BRAKE

abort the current robot motion

BREAK

delay program execution until the current motion completes; break continuous path motion CALIBRATE

initialize the robot positioning system CALL program_name(parameter_list)

transfer program execution to named subroutine

4

CALLS $string(parameter_list)

transfer program execution to named subroutine CASE value OF

multi-branch conditional statement providing a means for executing one of several groups of instructions [see VALUE(PD)]

CLOSE

close the hand during the next robot motion CLOSEI

cause a BREAK in the current motion, close the gripper, and delay "HAND.TIME" seconds COARSE ALWAYS

relax the positional tolerance necessary before the robot proceeds to the next location [see FINE(PI)]

CYCLE.END task_number, stop_flag

suspend execution until a specified program terminates (see CYCLE.END(MC)] DECOMPOSE array_name[index] = loc_variable

assign the components of a location to elements of a real array

DEFBELT %belt_variable = nominal_trans, belt_num,

vel_avg,scale_factor

define a belt variable

DELAY time

stop robot motion for a specified period of time DEPART distance

move to a location offset from the current location

DEPARTS distance

move in a straight-line to a location offset from the current location

DETACH (logical_unit)

release specified device from program control DEVICE (type, unit, error, p1, p2..1 ) out[], in[]

out_trans, in_trans

send commands and data to an external device [see DEVICES (PI), DEVICE (RVF)] DEVICES (type, unit, error, p1, p2,...) $out, $in

send commands and data to an external device [see DEVICE ( PI, RVF)} DISABLE switch, switch

****

turn off one or more system control switches [see DISABLE (MC)]

4

Adept Technology, Inc.

V and V+ Quick Reference Guide

5

DO...UNTIL condition

execute a group of instructions until a specified condition is met

DOS $string, error

execute a program instruction defined by a string expression

DRIVE joint, change, speed

move an individual joint of the robot

DURATION time ALWAYS

ELSE

set the time subsequent motions take to complete Isee DURATION (RVF)]

separate the alternate group of statements in an IF...THEN control structure

ENABLE switch,

END

switch

turn on one or more system control switches [see ENABLE (MC), DISABLE(PI,MC)]

mark the end of a control structure

[see CASE(PI), FOR(PI), IF(PI), WHILE(PI)]

EXECUTE task_num, program(parm_list), cycles, step,

slice, priority

begin execution of a program

FCLOSE (logical_unit)

close disk file or graphics window currently open on the specified logical unit

FCMND (logical_unit, command_code) out_string,

in_string_var

send a command to the logical unit

FDELETE (logical_unit) object

delete the specified disk file, graphics window, or graphics icon

FEMPTY (logical_unit)

empty internal buffer in use for a disk file or

graphics window by writing the buffer to the file or window if necessary

FINE ALWAYS

FLIP

specify normal positional tolerance before proceeding to the next location (default state) [see COARSE(PI)]

request change in wrist configuration during next motion [see NOFLIP(PI)]

*X

FOPEN (logical_unit) attribute_list

create and open a new graphics window, open an existing graphics window for access, or open a graphics icon for definition

FOPENA (logical_unit, rec_len, mode) file-spec open a disk file for read-write access FOPEND (logical_unit, rec_len mode) file-spec

open a disk for a directory read FOPENR (logical_unit, rec_len, mode) file-spec

open an existing disk file for read access FOPENW (logical_ unit, rec_len, mode) file-spec

open a new disk file for read-write access FOR index start TO stopSTEP increment

execute a group of instructions a specified number of times

FSEEK (logical_unit, record_number)

prepare the disk for the next access FSET (logical_unit) attribute_list

set or modify attributes of a graphics window GARC (logical unit,mode) xc,yc,radius,ang0,angn draw an arc or a circle in a graphics window GCHAIN (logical_unit) x, y, points, direction[index]

draw a chain of points to form a complex shape GCLEAR (logical_unit)

clear graphics window to the background color GCLIP (logical_unit) x, y, dx, dy

set clipping rectangle for all graphic commands except GFLOOD

GCOLOR (logical_unit) foreground, background

set the foreground and background colors for subsequent graphics commands

GCOPY (logical_unit) x, y - src_x, src_y, dx, dy

copy one region of a window to another region in the same window

GETEVENT (logical_unit, mode) events[index]

return input information from a graphics window GFLOOD (logical_unit) x, y

flood a region in a graphics window with color GGET.LINE (logical_unit) $data[index] = x,y,nx

flood a region in a graphics window with color GICON (logical_unit,mode) x, y, $icon_name, index

draw an icon in a graphics window

GLINE (logical_unit) x0, y0, xn, yn

draw a single line segment in a graphics window GLINES (logical_unit, mode) points, coord[offset,index]

draw multiple line segments in a graphics window

6

Adept Technology, Inc.

V and V+ Quick Reference Guide

7

GLOGICAL (logical_unit) code, planes

set logical operation to be performed between new graphics data and existing displayed graphics data; set which bit planes are affected

GOTO label

transfer execution to another step in the GPANEL (logical_unit, mode) x, y, dx, dy

program

draw a rectangular panel using shadowed or grooved edges

GPOINT (logical_unit) x, y

draw a single point in a graphics window GRECTANGLE (logical_unit, mode) x, y, dx, dy

draw a rectangular in a graphics window GSCAN (logical_unit) lines, data[offset,index]

draw a number of horizontal lines in a graphics window to form a complex figure

GSLIDE (logical_unit, mode) id x, y, len, max_pos,

arrow_inc, handle

draw slide bar in preparation for receiving slide

events

GTEXTURE (logical_unit) mode, pattern

set the opaque/transparent mode and the texture pattern for subsequent graphics output GTYPE (logical_unit, mode) x, y, $text, font, path, rotation

display a text string in a graphics window

HALT

stop program execution (cannot PROCEED or RETRY) [see PAUSE(PI), RETURN(PI),STOP(PI)]

HERE loc variable

assign a name to the current robot location [see HERE (TF,MC)]

IF condition GOTO label

conditionally branch to labeled line

IF condition THEN ...ELSE...END

control whether a group of instructions is executed

IGNORE signal

cancel the effect of a REACT or REACTI instruction

KEYMODE first_key, last_key = mode, setting

define operating characteristics of keys on the

manual control pendant

KILL task_number

clear the execution stack and detach any I/O devices attached by the specified task [see KILL(MC)]

*

LEFTY

request a left-handed configuration during the next motion [see RIGHTY(PI)]

LOCAL variable_list

declare variables to be defined only within the current program

LOCK priority

set the program reaction lock-out priority MC monitor command

introduce a monitor command within a command program

MCS $string

invoke a monitor command from a program

MOVE location

initiate a motion to specified location

MOVES location

initiate a straight-line motion to specified location

MOVET location, hand_opening

initiate a motion to specified location and simultaneously operate the hand

MOVEST location, hand_opening

initiate a straight-line motion to specified location and simultaneously operate the hand

MULTIPLE ALWAYS

allow full rotation of the wrist joints (default state) [see SINGLE(PI)]

NOFLIP

request change in wrist configuration during next motion [see FLIP(PI)]

NONULL ALWAYS

disable position-error checking at the end of continuous-path motions

NULL ALWAYS

enable position-error checking at the end of continuous-path motions (default state)

OPEN

open the hand during the next robot motion OPENI

?

cause a BREAK in the current motion, open the gripper, and delay "HAND.TIME" seconds

PACK string array[index], first_char, num_chars = string

replace a substring within an array of string variables

PACK string _var, first_char, num_chars = string

replace a substring within a string variable

8

Adept Technology, Inc.

V and V+ Quick Reference Guide

9

PARAMETER parameter_name = value

set the value of a system parameter [see PARAMETER (MC,RVF)]

PAUSE

suspend program execution (until PROCEED is entered)

.PROGRAM program_name(argument_list)

define the arguments a program will recieve and

return

PROMPT $output_string, variable_list

display output and request input from the keyboard REACT signal_number, program_name, priority

initiate monitoring of a specified digital signal (1001-1016 or 2001-2008) and call a subroutine if the signal transitions

REACTE program_name

initiate or cancel monitoring of errors during program execution REACTI signal_number, program_name, priority

initiate monitoring of a specified binary signal (1001-1016 or 2001-2008) and if it transitions, stop the arm motion and optionally call a subroutine READ (logical_unit, rec_num, mode) variable_list

read a record from an open file or from an attached device

READY

move the robot to a standard, pre-defined location/configuration

RELAX

turn off the air to the gripper during the next robot motion

RELAXI

cause a BREAK in the current motion, turn off the air pressure to the gripper and delay "HAND.TIME" seconds

RELEASE

allow the next available program task to run [see WAIT (PI)]

RESET

turn off all the digital output lines [see RESET (MC)] RETURN

transfer execution from a subroutine back to the calling program

RETURNE

transfer execution from an error subroutine to the step following the instruction causing the error

02.

RIGHTY

request a right-handed configuration during the

next motion [see LEFTY(PI)]

RUNSIG signal_number

turn on/off a digital signal until program execution terminates

SEE (logical_unit) program_spec, step

initiate screen-oriented editing of a program

[see SEE(MC)]

SELECT device_type = unit

select a device for access by the current program

task [see SELECT (MC, RVF)]

SET loc_variable = loc_value

define a location variable equal to the value given SETBELT %belt variable = expression

synchronize the belt encoder

SETDEVICE (type, unit, error, command) p1,p2,...

initialize or set parameters for an external device SIGNAL signal_number,....., signal_number

turn on/off external digital output lines or internal software signals [see SIGNAL (MC)] SINGLE ALWAYS

limit wrist joint rotations to a range of ± 180 degrees [see MULTIPLE(PI)]

SOLVE.ANGLES o.jts[o.idx], o.flags, error = trans,

i.jts[i.idx], i.flags

compute the robot joint positions that are equivalent to a specified transformation SOLVE.TRANS transform, error = joints[index]

compute the transformation equivalent to a specified array of joint positions

SPEED speed factor, rot_spd_factor units ALWAYS

set the nominal speed for subsequent motions [see SPEED (MC,RVF)]

STOP

terminate execution of the current program cycle SWITCH Switch_name = value

enable or disable a system switch based on a value [see SWITCH (MC,RVF)]

TIME $time_string

set the date and time [see TIME (MC,SF,RVF)] (e.g."20-MAY-91 16:43")

TIMER timer number = time value

set the specified system timer (1-15) to the given time value [see TIMER (RVF)]

10

Adept Technology, Inc.

V and V+ Quick Reference Guide

11

TOOL transform_value

define the robot tool configuration, relative

to the tool mounting flange [see TOOL (TF,MC)]

TYPE output_spec, output_spec,...

display information on the terminal

VALUE expression, expression...:

indicate values that a CASE statement can match [see CASE(PI), ANY(PI)]

variable

value

assign a variable name to the value on the right side of the expression

VAUTOTHR (dmode, start, end) array[index] = frame

automatically determine thresholds for binary image processing based on the image last VPICTURE'd [see VAUTOTHR (MC)] VCONVOLVE (type) dest_frame = src_frame

perform an image convolution on a grayscale frame

VCOPY dest_frame = source_frame

copy image from one frame to another VDEFGRIP $proto, grip, mode, num_fingers,

trans[i],w[j],h[k]

define a gripper shape/position for clear-grip tests a gripper number of zero deletes all defined grips for the specified prototype

VDEF.CONVOLVE type = array[,]

www

define an image convolution

VDEF.FONT (op) font_num, $chars, ht, black_chars

define, replace, or modify an OCR font

VDEF.MORPH type = array[]

define a binary morphological operator VDEF.SUBPROTO proto:subname, first_edge, last_edge

define sub-prototype [see VDEF.SUBPROTO (MC)]

VDELETE model_name

delete a specified prototype or its sub-prototype or an OCR font from vision memory

VDISPLAY(camera) mode, overlay

select the vision monitor display mode to be

used while the vision system is performing its normal processing functions [see VDISPLAY (MC)] VEDGE (type, thresh) frame

compute edges in image and threshold the edges using either a cross gradient or Sobel operator VEDGE.INFO data[index] = $name, edge_num

retrieve information about the edges and corners of a prototype or region in the image

[see V.EDGE.INFO(SW)]

VFIND.ARC (cam,mode, dmode,effort) data[i] = xc,yc, r, rr,

ang0,angn

fit a circular arc to an image edge bounded by a window that is shaped like a ring or a ring segment VFIND.LINE (cam, pos, dmode, effort) data[i] = xc, yc, len,

wid, ang

fit a straight line to an image edge bounded by a window

VFIND.POINT (cam, pos, dmode) data[i]

ang

= xc, yc, len,wid,

find an edge point in a search window that is nearest to one side of the window

VGAPS data[i] = proto_name, edge_num

find the unverified gaps in a match with a prototype or sub-prototype

VGETCAL (camera) array[index]

fill an array with the previously defined vision calibration data for a given virtual camera

VGETPIC (cam, type, s_rate, s_mode) $pic[r,c] = shape, x0,

y0, dx, dy

read all or part of an image into a string array VHISTOGRAM (dmode) array[index] = frame

compute the histogram for a grayscale frame store [see VHISTOGRAM(MC)]

VLOAD (logical_unit) $file_spec

load vision prototypes or OCR fonts from a disk file [see VLOAD (MC)]

VLOCATE (camera, mode, order) $name, trans_variable

identify and locate an object in the scene VMORPH (type) frame

perform a morphological transform on a binary image frame

VOCR (cam, op, dmode) data[i], locs[j] = font_num,

$expected, shape, cx, cy, dx, dy, ang

perform OCR recognition or text verification in a rectangular image window

VPICTURE (cam, wait, grab_frame, sel_frame) mode,

how many

grab an image for processing into "grab_frame"

buffer [seeVPICTURE (MC)]

VPUTCAL (camera) array[index]

fill an array with vision calibration data

VPUTPIC (camera, type, zoom) Spic[r,c], x0, y0

store into a frame store an image saved previously with VGETPIC

12

Adept Technology, Inc.

V and V+ Quick Reference Guide

13

VRULERI (cam, type, dmode,maxcnt) data[i], mags[j]=

shape, x0, y0, len, ang

obtain edge information or gray levels along a line in the current image

VRULERI (cam, type, dmode,maxcnt) data[i], mags[j] =

shape, cx, cy, radius, ang0, angn

obtain edge information or gray levels along a circular arc in the current image

VSELECT frame_store

select one of two grayscale frame stores for processing or display

VSHOW mode, $proto_name, trans_variable, grip,

edge_num

display a vision prototype or sub-prototype and make information about it available through the VFEATURE function [see VSHOW (MC)] VSHOW.MODEL (mode) $chars, data[i] = $font_num

display one or all OCR fonts and return information about them

VSTATUS (camera, type) array[i]

return vision system status information in a real array [see VSTATUS (MC)]

VSTORE (logical_unit) $file_spec = $model_name, ..

$model_name

store selected (or all) vision prototypes (and their sub-prototypes) or OCR fonts in a disk file [see VSTORE (MC)]

VSUBPROTO ver_percent, trans_variable = subproto_name,

edge_num

determine the percentage of an edge or sub- prototype that was verified during recognition VSUBTRACT (type) dest_frame = srcl_frame,

src2_frame

subtract two binary or grayscale images

VTHRESHOLD (type, thr1, thr2) frame

threshold a grayscale image, producing a binary image

VTRAIN prototype, shape, cx, cy, width, height, ang

initiate prototype training of the named object [see VTRAIN (MC)]

VTRAIN.MODEL (cam, plan) $font_num, $text, shape, cx,

cy, dx, dy, ang

train on a font or perform OCR planning

VWAIT

delay program execution until a VPICTURE or VWINDOW operation is complete.

ย

VWINDOW (cam, type, dmode, how_many) shape, cx, cy,

wid, ht, ang

perform image processing in a rectangular window in the image

VWINDOWI (cam, type, dmode, sample) data[i] = shape,

cx,cy, dx, dy, ang

extract image data from a rectangular window VWINDOWI (cam, type, dmode, sample) data[i] =shape, cx,

cy, or, ir, ang0,angn

extract image data from a circular, ring, or pie-shaped window

WAIT condition

put program execution into a "wait state" until the condition is TRUE, or for one system clock tick WHILE condition DO

execute a group of instructions as long as a

specified condition is met

WINDOW %belt_variable - loc_variable, loc_variable,

program, priority

define a working window on a conveyor belt [see WINDOW (RVF)]

WRITE (logical_unit, record_number, mode) ouput_specs write a record to an open file, or to an attached device via a logical unit number

14

Adept Technology, Inc.

V and V+ Quick Reference Guide

15

MONITOR COMMANDS

ABORT task__number

terminate execution of a control program

BASE X_shift, Y_shift, Z_shift, Z_rotation

define the location of the robot base, relative to its

workspace [see BASE (TF,P1)]

BITS first_signal, num_sigs = value

www.TA

set the state of a group of digital output or software signals [see BITS (RVF,PI)]

BPT @task:program step (expression_list)

set or clear breakpoint for debugging

CALIBRATE

initialize the robot positioning system COMMANDS program_name

initiate processing a monitor command program [see MC (PI)]

COPY new_program_name = source program__name create new program as a copy of an existing program

CYCLE.END task_number, stop_flag

suspend execution until a specified program terminates [see CYCLE.END(PI)}]

DEBUG task_number program_name

initiate the screen-oriented program debugger

DEFAULT logical_device =

physical_device>unit: \directory_path\

set the default relationship between a logical device and a physical device

DELETE program_name,..., program_name

delete specified programs and all associated data

DELETEL @task:program loc_variable_list

delete the specified location data

DELETEM module_name

delete a program module and all of its programs

DELETEP program_name, ...,program_name

delete the specified programs

DELETER @task:program real_variable_list delete the specified real variables DELETES @task:program string_variable_list delete the specified string variables

DIRECTORY select

display of the names of some or all programs in memory

DISABLE switch, switch

16

turn off one or more system control switches

Adept Technology, Inc.

<

DO @task:program instruction

execute a single program instruction as though it were the next step in the current main control program, or the next step in the specified task/ program context

EDIT program_name, step

initiate line-oriented editing of a new or existing program

ENABLE switch, ...,

switch

turn on one or more system control switches [see ENABLE (PI)]

EXECUTE task_num, program(parm_list), cycles, step, slice,

priority

begin execution of a program [see EXECUTE(PI)] FCOPY new_file_name = source_file_name

copy an existing file into a new file

FDELETE physical_device>disk_unit:\dir\file_spec delete a file from the disk [see FDELETE(PI)] FDIRECTORY physical_device>disk_unit:\dir\file_spec

display a list of files on disk FDIRECTORY/qualifier directory_spec

create or delete subdirectory on a disk

FLIST physical_device>disk_unit:\dir\file_spec

display the contents of a disk file on the terminal FORMAT disk_unit: /qualifier

erase a disk and initialize the file structure FREE select

display the percentage of available system or graphics memory [see FREE (RVF)]

FRENAME new_file_name = old file name

change the name of a disk file

(disk or directory specification, if given, must be on left)

HERE @task:program loc_variable

assign a name to the current robot location

[see HERE (TF,PI)]

ID device

display identity information about components of the system[see ID (RVF)]

10 signal_group

display the states of the digital signal lines

KILL task_number

clear the execution stack and detach any I/O devices attached by the specified task [see KILL(PI)] LISTL @task:program location,...., location

display location data specified

V and V+ Quick Reference Guide

17

LISTP program,

program

display the contents of the programs specified LISTR @task:program real_expression,..., real_expression

display numeric item(s) specified

LISTS @task:program $string,..., $string

display string variables or expressions specified LOAD physical device>disk_unit:\dir\file_spec

load program into memory from disk file MDIRECTORY module_name

display names of program modules in system memory, or programs in a module MODULE module_name = program,...,program

NET

create a new program module, or modify the contents of an existing module

display operational statistics for DDCMP communications lines

PANIC

stop robot motion immediately

PARAMETER parameter_name

value

display or set the value of system parameters

[see PARAMETER (PLRVF)]

PASSTHRU port_number

provide a direct connection between the system terminal and a "USER" serial port on the controller POINT @task:program_loc_var = loc_value

define a location variable

PRIME task_number program_name (parameter_list), cycles,

step, slice, priority

prepare for execution of a program

PROCEED task_number

resume execution of a program

RENAME new_name old_name

change the name of a program in memory

RESET

turn off all the digital output lines [see RESET (PI)] RETRY task_number

repeat execution of last interrupted program instruction and continue execution of program

SEE program_name [R

initiate screen-oriented editing of a new or existing program [see SEE(PI)]

SELECT device_type = unit

select a device for access by the current program task [see SELECT (PI, RVF)]

SIGNAL signal_number,....., signal_number

turn on/off external digital output lines or internal software signals [see SIGNAL (PI)]

SPEED value

set the overall speed of robot motions

[sce SPEED (PIRVF)]

SSTEP task number

execute one step or a subroutine of a program

STACK task_ number

size

specify amount of system memory reserved for the stack of a program task

STATUS select

display system information and program status STORE physical_device>disk_unit:\dir\file_spec =

program,...., program

store programs and variables from memory into a disk file

STOREL physical_device>disk_unit:\dir\file_spec =

program,...., program

store referenced location variables from memory into a disk file

STOREM physical_device>disk_unit:\dir\file_spec

module_name

store programs from memory into a disk file STOREP physical_device>disk_unit:\dir\file_spec

program,...., program

store programs from memory into a disk file STORER physical_device>disk_unit:\dir\file_spec =

program...., program

=

store referenced real variables from memory into a disk file

STORES physical_device>disk_unit:\dir\file_spec =

program,...., program

store referenced string variables from memory into a disk file

SWITCH switch_name[index]

display the settings of system switches

[see SWITCH (PLRVF)]

TEACH @task:program_loc_variable

initiate recording of locations using the manual control pendant

TESTP program_name

test for presence of the named program in memory TIME dd-mmm-yy hh:mm:ss

set/display date and time [see TIME (PI,RVF,SF)] (e.g., TIME 20-MAY- 90 16:43)

18

Adept Technology, Inc.

V and V+ Quick Reference Guide

19

TOOL @task:program transform_value

define the robot tool configuration, relative to tool mounting flange [see TOOL (TF,PI)] VAUTOTHR (dmode, start, end) array[index] = frame

automatically determine thresholds for binary image processing based on the image last VPICTURE'd [see VAUTOTHR (PI)]

VDEF.SUBPROTO proto:subname, first_edge, last_edge

define a sub-prototype [see VDEF.SUBPROTO (PI)] VDELETE model_name

delete a specified prototype, sub-prototype, or OCR font [see VDELETE (PI)]

VDELETE font_num

delete a specified font (ie. FONT_2)

[see VDELETE (PI)]

VDISPLAY (camera) mode, overlay

select the vision monitor display mode to be

used while the vision system is performing its normal processing functions [see VDISPLAY (PI)] VHISTOGRAM (dmode) array[i] = frame

compute a histogram for a grayscale frame store [see VHISTOGRAM(PI)]

VLOAD physical_device>disk_unit:\dir\file_spec

load vision prototypes or OCR fonts from a disk file VPICTURE (cam, wait, grab_frame, sel_frame) mode,

how many

grab an image for processing into "grab_frame" buffer [seeVPICTURE (PI)]

VQUEUE (camera)

display information for any objects queued in the vision system, awaiting retrieval by VLOCATE instructions

VRENAME new_name = old name

rename a prototype or sub-prototype

VSHOW proto_name, grip, edge_num

list the defined prototypes or display a vision

prototype, a sub-prototype, or a specific edge on the monitor; edge numbers are optionally shown VSTATUS

display vision status information on the monitor VSTORE physical_device>disk_unit:\dir\file_spec =

proto_name

proto_name,

store selected (or all) vision prototypes (and their sub-prototypes) or OCR fonts in a disk file

[see VSTORE(PI)]

VTRAIN prototype, shape, cx, cy, width, height, ang

initiate training of object named [see VTRAIN(PI)] WAIT.START condition

put a monitor command program into a "wait loop" until a condition is TRUE, or until PROGRAM START is pressed

WATCH @task:program expression

set or clear watchpoint for debugging

WHERE mode

display the current robot location

XSTEP task_number program(parameter_list), cycles, step,

slice, priority

execute a single step of a program

ZERO select

erase the contents of selected system memory

20

20

Adept Technology, Inc.

V and V+ Quick Reference Guide

21

ABS(value)

FUNCTIONS

Real-Valued Functions

absolute value (magnitude)

ACCEL(select)

return the acceleration or deceleration setting ASC($string, index)

convert a character into its ASCII value ATAN2(value_1, value_2)

arctangent function, given two sides of a right triangle

BCD(value)

convert from real-value representation to Binary Coded Decimal

BELT(%belt_variable, mode)

return the instantaneous belt position or speed BITS(first_signal, num_sigs)

read the state of a group of digital lines, interpreting the result as a value [see BITS (MC,PI)}

BSTATUS

return bit flags indicating the status of conveyor tracking

CONFIG(select)

return a value that provides information about the robot's geometric configuration, or the status of the motion servo-control features

COS(value)

cosine function (input parameter in degrees) DCB(value)

convert BCD representation to an integer value DEFINED(variable_name)

determine if a variable is defined

DEVICE(type, unit, error, p1, p2, ...)

return data from an external device

[see DEVICE (PI), DEVICES (PI)]

DISTANCE(location_1, location_2)

determine the distance between two locations DURATION(select)

return the current setting of one of the motion DURATION specifations [see DURATION (PI)] DX(location)

return X-axis component of a location

DY(location)

22

return Y-axis component of a location

Adept Technology, Inc.

DZ(location)

return Z-axis component of a location

ERROR(task, select)

determine the error number of the latest error

[see $ERROR (SF)]

FALSE

return a logical "false" (value = 0)

FLTB(string, first_char)

return the value of four bytes of a string interpreted

as an IEEE single-precision floating-point number [see $FLTB (SF)]

FRACT(value)

return the fractional part of the argument

FREE(memory, select)

return information about available system or graphics memory [see FREE (MC)]

GETC(logical_unit, mode)

get next character (byte) from a device or input record via a logical unit

HAND

determine the current hand opening

HOUR.METER

return the value of the robot power-on meter

(for Adept robots equipped with Remote Signature Cards only)

ID(component, device)

return a value that identifies the current system configuration [see ID (MC)]

IDENTICAL(location 1, location 2)

determine if two locations are exactly the same INRANGE(location)

determine if a location can be reached

INT(value)

integer portion of the argument, with the sign INTB($string, first_char)

return integer value from a 2-byte string [see $INTB (SF)]

IOSTAT(logical_unit, mode)

provide status information on the most recent 1/0 operation for the specified logical unit

LAST(array_name[])

return highest index used for an array (dimension) LATCHED(

return the status of the external trigger, and of the information it causes to be "latched"

V and V+ Quick Reference Guide

23

LEN($string)

return the length of a string

LNGB ($string, first_char)

return value of 4 bytes of a string interpreted as a

signed 32-bit integer [see $LNGB (SF)]

MAX(value,..., value)

return maximum value in a list of values

MIN(value,...,value)

OFF

ON

return minimum value in a list of values

return the value 0 (synonym for FALSE)

return the value -1 (synonym for TRUE) PARAMETER(parameter_name)

return the current setting of a system parameter [see PARAMETER (MC,PI)]

PENDANT(select)

PI

return data from the manual control pendant

return the value of the constant "" (3.141593) POS($search_string, $sub string, start)

return the character position of a substring PRIORITY

return the current program priority as set by a LOCK, REACT or REACTI instruction

RANDOM

return a pseudo random number

SELECT(name, mode)

for a named device, return unit number currently selected by the current task, or the total number of units available [see SELECT (PI, MC)]

SIG(signal_number,..., signal_number)

return a logical AND of the states of specified digital lines

SIG.INS(signal_number)

check if a digital line is configured or if a software signal is available

SIGN(value)

return the sign of the argument

SIN(value)

sine function (input parameter in degrees) SOLVE.FLAGS(joints[index])

return bit flags representing the robot

configuration specified by an array of joint positions

SPEED(select)

return one of the system motion speed factors [see SPEED (MC,PI)]

SQR(value)

return the square of the value

SQRT(value)

return the square root of the value

STATE(select)

return a value that provides information on the

current system state

STATUS(program_name)

return status information for an application

program

SWITCH(switch_name)

return the state of a system switch

[see SWITCH (MC,PI)}

TAS(variable, new_value)

return the current value of a real-valued variable

and assign it a new value

TASK(select, task_number)

return information about a program task

TIME(string, select)

return an integer value representing a date or time [see TIME (MC,PI,SF)]

TIMER(timer_number)

TPS

return the value (in seconds) of the specified timer [see TIMER (PI)]

return the number of system clock cycles (ticks) occurring per second

TRUE

return a logical "true" (value -1)

VAL(string)

return a real value represented by the characters in the input string

VFEATURE(index)

return specified information about the object most recently VLOCATE'd or the proto most recently displayed by the VSHOW program instruction

VQUEUE(camera, $name)

return the number of objects in the vision queue WINDOW(compound_transformation, time, mode)

indicate where a location is in relation to a window during belt tracking [see WINDOW (PI)]

24

Adept Technology, Inc.

V and V+ Quick Reference Guide

ન

25

#PLATCH()

Precision-Point Functions

return a precision-point value representing the location of the robot at the occurrence of the last external trigger

[see LATCH(TF), LATCHED(RVF)]

$TRUNCATE($string)

return characters in the input string until an ASCII NUL or end of string is encountered $UNPACK(string_array[index], first_char, num_chars)

return a substring from a string array

Transformation Functions

$CAL.DATA(index)

String Functions

return the calibration tables for the robot currently selected

$CHR(value)

convert a numeric value into a character $DECODE($string_variable, $string_expr, mode)

remove a substring from the given string variable $DEFAULT( )

return a string containing the current system default device, unit, and directory path for disk file access [see DEFAULT(MC)]

SENCODE(output_spec,..., output _spec)

produce a string from strings, expressions, real variables and/or format specifiers

$ERROR(error_code)

return the error message for an error code value [see ERROR (RVF)]

$FLTB(value)

return the 4-byte binary representation of a floating-point value [see FLTB (RVF)]

$INTB(value)

return the 2-character binary representation of a numeric value [see INTB (RVF)]

SLNGB(value)

return a 4-byte string containing the binary representation of a 32-bit integer [see LNGB (RVF)]

SMID($string, first_char, num_chars)

return a substring from the specified string

$TIME(date, time)

return a string containing date and time information [see TIME (MC,PI,RVF)]

$TRANSB(transformation_value)

return a 48-byte string representation of a transformation value [see TRANSB (TF)]

BASE

return the current base location [see BASE (MC,PI)]

DEST

return the planned destination transformation for the current motion

FRAME(loc_1, loc_2, loc_3, loc_4)

compute a reference frame based on the given locations

HERE

return the current robot location [see HERE(MC,PI)] INVERSE(transformation_value)

return the inverse of the specified transformation

LATCH()

return a transformation value representing the location of the robot at the occurrence of the last external trigger

[see LATCHED(RVF), #PLATCH(PPF)] NORMAL(transformation_value)

correct a transformation for mathematical round-off errors

NULL

return the null transformation

#PPOINT(j1_val, j2_val, j3_val, j4_val, j5_val, j6_val)

compose a precision point from individual joint components

RX(angle)

return a transformation describing a given

rotation in degrees about the X-axis

RY(angle)

return a transformation describing a given

rotation in degrees about the Y-axis

RZ(angle)

return a transformation describing a given

rotation in degrees about the Z-axis

SCALE (transform_value BY scale_factor)

scale the translation portion of a transformation by the specified factor

26

26

Adept Technology, Inc.

V and V+ Quick Reference Guide

27

SHIFT(transform_value BY x_shift, y shift, z__shift)

shift the translation portion of a transformation TOOL

return the current tool definition[see TOOL(MC,PI)] TRANS(x_val, y_val, z_val, yaw_val, p_val, r_val)

compose a transformation from individual values TRANSB($string, first_char)

return a transform represented by a 48-byte string [see $TRANSB (SF)]

BELT

CP

SWITCHES

control use of the conveyor tracking feature

control continuous-path motion through locations DRY.RUN

control whether or not V/V+ communicates with

the robot

FORCE

control the optional stop-on-force feature INTERACTIVE

control certain interactions with the terminal

MCP.MESSAGES

control how system error messages are handled when the controller key switch is not in the "PENDANT" position

MCS.MESSAGES

control terminal output from commands invoked with MCS instructions

MESSAGES

control output to the system terminal from TYPE instructions

POWER

control status of robot power

RETRY

control whether or not the PROGRAM START button causes a program to resume

SET.SPEED

control setting of monitor speed from pendant TRACE

enable each program step of task 0 to be displayed as it is executed

UPPER

control case conversion during string comparisons VISION

enable the optional vision system

V.2ND.MOMENTS[camera]

enable computation of the best-fit ellipse for each region in the image

V.BACKLIGHT[camera]

establish the color (black or white) of the background

28

Adept Technology, Inc.

V and V+ Quick Reference Guide

29

V.BINARY[camera]

select binary image processing or grayscale edge processing

V.BOUNDARIES[camera]

enable boundary analysis by the vision system V.CENTROID[camera]

enable centroid computation of each region V.DISJOINT[camera]

specify whether or not prototypes may be matched to multiple, disjoint regions

V.DRY.RUN[camera]

when enabled, the inspection and measurement operators only do graphics

V.EDGE.INFO[camera]

enable collection of information for use by the VEDGE.INFO instruction

V.FIT.ARCS(camera]

control whether or not arc fitting is done by VPICTURE and VWINDOW operations

V.HOLES(camera]

enable the accounting of interior features in all objects

V.MIN.MAX.RADII[camera]

enable the feature that, for each region in the image,

finds the two points on the perimeter that are closest to and farthest from the region centroid

V.OVERLAPPING[camera]

determine whether or not objects may be

overlapping in the image

V.PERIMETER[camera]

enable computation of the lengths of region perimeters

V.RECOGNITION[camera]

enable object recognition by the vision system V.SHOW.BOUNDS[camera]

enable the special display of the lines/arcs fit to the boundaries of regions

V.SHOW.EDGES[camera]

enable the special display of the primitive edges that are fit to the boundaries of regions

V.SHOW.GRIP[camera]

enable the special display of clear-grip tests

V.SHOW.RECOG[camera]

enable the special display of the objects recognized

V.SHOW.VERIFY[camera]

enable the special display of the verification steps in the recognition process

V.STROBE(camera)

enable the firing of a strobe light in synch with VPICTURE operations

V.SUBTRACT.HOLE[camera]

determine whether hole areas are to be subtracted from region areas

V.TOUCHING[camera]

determine whether or not objects may be touching in the image

30

Adept Technology, Inc.

V and V+ Quick Reference Guide

31

BELT.MODE

PARAMETERS

control operation of the conveyor tracking feature (bit flags, default = 0)

BELT.ZERO.COUNT

set number of encoder pulses between zero-index pulses for all belt encoders used (default = 4000, value of 0 disables all belt servo errors) DISPLAY.CAMERA

control the display of system parameters and switches on the terminal by specifying how many virtual cameras used (0 to 32, default = 4) HAND.TIME

specify the duration of the motion delay when processing OPENI, CLOSEI or RELAXÍ

(0 to 1 x 10 seconds, default = 0.05 seconds) KERMIT.RETRY

specify maximum number of times V+ Kermit driver should retry an operation before reporting an error (1 to 1000, default = 15)

KERMIT.TIMEOUT

specify the delay parameter that the V+ Kermit driver for the Kermit protocol will send to the re- mote server (1 to 95, default = 8) NOT.CALIBRATED

indicate (or assert) the calibration status of the robots connected to the system

TERMINAL

specify how the V/V+ interacts with the system terminal (0 to 4, default = 4)

V.2ND.THRESH[camera]

set a second threshold for use during binary image processing (0 to 127, default = 0)) V.BORDER.DIST[camera]

define an image border reduction (in pixels) to mask out regions clipped by the image border (0 to 100 pixels, default -- 0)

V.EDGE.STRENGTH[camera]

set the edge threshold for grayscale processing and fine-edge rulers (0 to 127, default = 20) V.EDGE.TYPE(camera)

select type of edge operator to use with VEDGE or VPICTURE instructions (1 or 2, default = 1)

V.FIRST.COL[camera]

set the first column of camera data to be processed (1 to 512, default = 1)

V.FIRST.LINE[camera]

set the first line of area camera data to be processed

(1 to 484, default = 1)

V.GAIN[camera]

set the gain for the incoming video signal

(1 to 256, default = 128)

V.IO.WAIT[camera]

enable synchronization of image acquisition (VPICTURES) with an external event that triggers the fast digital I/O interrupt line

(0 to 1, default =0)

V.LAST.COL[camera]

set the last column of camera data to be processed (1 to 512, default = 512)

V.LAST.LINE[camera]

set the last line of area camera data to be processed (1 to 484, default = 484)

V.LAST.VER.DIST[camera]

enable an extra verification of prototype-to- image matches and specify the pixel tolerance

to use when determining boundary coincidence (0 to 16, default = 0)

V.MAX.AREA[camera]

set the maximum area (in pixels), above which the vision system ignores regions

(1 to 1048576, default = 262144)

V.MAX.PIXEL.VAR[camera]

set maximum pixel deviation allowed when fitting lines and arcs during boundary analysis

(0.0 to 8.0, default = 1.5)

V.MAX.TIME[camera]

set the maximum time (seconds) allowed to analyze

a region (1 to 999, default = 5)

V.MAX.VER.DIST[camera]

set the pixel tolerance for determining boundary coincidence during the verification of proto-to-image matches (1 to 16, default = 3) V.MIN.AREA[camera]

set the minimum area (in pixels), below which the system ignores regions (1 to 1,048,576, default = 16)

32

Adept Technology, Inc.

V and V+ Quick Reference Guide

33

V.MIN.HOLE.AREA[camera]

set minimum area (in pixels), below which the

system ignores holes

(1 to value of V.MIN.AREA, default = 8)

V.OFFSET(camera]

set the offset for the incoming video signal

(0 to 255, default = 127)

V.THRESHOLD[camera]

set the camera grayscale value that separates "black" and "white" pixels (0 to 127, default = 63)

+- */

MOD

<

V VAIA A

IV

<>

>=

AND NOT

OR

XOR

OPERATORS

Mathematical Operators

arithmetic addition, subtraction,

multiplication and division

modulus (remainder after integer division)

Relational Operators

less than

less than or equal equal

not equal

greater than or equal

greater than

Logical Operators

logical AND of two TRUE or FALSE values logical complement of a TRUE or FALSE value logical OR of two TRUE or FALSE values logical exclusive OR of two TRUE or FALSE values

Binary Operators

bit-wise OR of two integer values

BAND BOR

bit-wise AND of two integer values

BXOR

COM

bit-wise exclusive OR of two integer values

bit-wise complement of an integer value

Note: Order of Evaluation

1) Functions are evaluated

2) NOT and COM operators are processed left to right

3) Unary minus operators are processed

4) Multiplication, division, modulus, AND and BAND

operators are processed from left to right

5) Addition, subtraction, OR, BOR, XOR and BXOR operators are processed from left to right

6) Relational operators are processed from left to right

String Operator

+

string concatenation

34

Adept Technology, Inc.

V and V+ Quick Reference Guide

35

NOTES

Floppy Disk FORMAT Command Qualifiers:

36

Format

Total

IBM PC

Qualifier

Sectors

Compatible

/S

320

5.25"

/D

640

No

1Q

1280

No

/I

720

5.25"

/G

1440

No

/H

1440

3.5"

Adept Technology, Inc.